# A Stage-Law Model of the IBIS Predriver: Derivation, Fitting and Verification

October 15, 2026

# II. The Stage-Law Buffer Model

This section presents the stage law, the model of the predriver used in this work, derived from the α-power law MOSFET model [1], and the buffer model built on it. The preliminaries state the problem and the terms; Sections II-A to II-E derive the law; Sections II-F to II-O build the buffer model, fit it to an IBIS file and one measurement, and verify it. Equations are numbered consecutively, the approximations are summarized in Section II-O, and notation is collected in Table I.

## Preliminaries: the problem, the idea and the terms

What an IBIS file contains. An IBIS file describes an output buffer from its pins. For the driver it holds three things: I–V tables, which give the current the output transistors deliver against pad voltage with the pull-up fully on (and likewise the pull-down); V–T waveforms, which give the pad voltage against time for one complete rising and one complete falling transition into a stated load; and the die capacitance Ccomp with the package parasitics. The standard way to simulate from such a file is to solve, from the V–T waveforms, two time-varying coefficients Ku(t) and Kd(t), the fraction of the pull-up and of the pull-down table that is conducting at each instant, such that KuIPU+KdIPD reproduces the recorded waveforms. In simulation the coefficients are then replayed against time from the input edge. This is what the document calls the existing converter, and “the file's Ku(t)” means these coefficients.

The problem. Those coefficients describe complete transitions, and that is what the file records (Fig. 1(a)). When the input pulse is shorter than the buffer's own delay and edge, the transition is cut short: in Fig. 1(b) an 810 ps pulse drives ex2's pad to 0.77 V of its 1.55 V swing, and the model replayed from the file overshoots the peak by 71 %. On the twelve test buffers of this work the error of such a model is 20–100 % (Table XVII). Physically the pulse is interrupted inside the predriver, the chain of inverters between the input pin and the output transistors, while it is still travelling: the gate of the output transistor turns back partway, and a coefficient replayed against time has no way to represent that.

Fig. 1. The problem, on test buffer ex2. (a) A complete transition into the test load: the situation an IBIS file records. (b) An input pulse of 810 ps, shorter than the buffer's delay and edge: the transistor-level pad voltage against HSPICE's native IBIS model of the same file, which replays the file's coefficients against time.

The idea. Model the predriver as a chain of stages whose output is the gate of the output transistor; convert that gate into Ku and Kd through a static map, so that the fractions follow the gate rather than the clock; fit the chain's rates to the Ku(t) that the file already contains; and use one measured short pulse to settle what the file cannot. Sections II-A to II-E derive the stage law from the transistor. Sections II-F to II-O build the buffer model from it, fit it, and verify it at the gate and at the pad.

Table II: Terms used throughout

| Term | Meaning |
| --- | --- |
| the file | the IBIS file of the buffer being modeled |
| the existing converter; native model | the IBIS-to-SPICE converter this work extends, which replays the file's Ku(t), Kd(t) against time; HSPICE's own IBIS element does the same |
| pad; die node | the buffer's output pin; the node inside the package where the output transistors and Ccomp connect |
| complete (full) transition; settled swing | an input edge the buffer finishes before the next one; the pad voltage it reaches |
| stressed pulse; depth | an input pulse too short for the transition to complete; the fraction of the settled swing the pad reaches (the tests use about 50–90 %) |
| predriver; stage | the chain of inverters between the input pin and the output transistors; one inverter of it |
| output device; gate | the pull-up (or pull-down) transistor that drives the pad; its gate node, written g or GUP on a 0-to-1 scale |
| Ku, Kd; conducting fraction | the multiplier on the pull-up (pull-down) I–V table at a given instant |
| the map M | the fixed curve giving Ku as a function of the gate |
| candidate | one complete model: a stage count, a map shape and the stage numbers fitted for them |
| probed | read from an internal node of the transistor netlist, which only the development tests can do |

Three thresholds and three exponents appear and should not be confused. VTH is the device threshold of Section II-B; vt is the stage threshold, defined from VTH in (9) but treated from Section II-J onward as an effective, fitted number; vt,map is the gate level at which the output device starts to conduct in the map. α is the device's velocity-saturation index, p its stand-in in the stage law (fixed at 1), and a the curvature of the map. Table I at the end collects every symbol.

## A. Node dynamics

A predriver stage is a CMOS inverter whose output node, at voltage Vo, drives a lumped capacitance C. Kirchhoff’s current law at that node gives

C dVodt=IP−IN    1

where IP is the pull-up (PMOS) current into the node and IN is the pull-down (NMOS) current out of it. Equation (1) is the only physical law used; all remaining steps model IP and IN and change variables.

## B. Device model

Each transistor is described by the α-power law [1]. For a device with gate–source voltage VGS and drain–source voltage VDS (magnitudes),

ID=0,D=0 cutoffI′D0V′D0 VDS,VDS<V′D0 triodeI′D0,VDS≥V′D0 saturation    2

with

I′D0=ID0 Dα    3

V′D0=VD0 Dα/2    4

D=clipVGS−VTHVDD−VTH, 0, 1    5

Here ID0 and VD0 are the saturation current and saturation voltage at VGS=VDD, VTH is the threshold, and α∈1,2 is the velocity-saturation index (α=2 recovers the long-channel square law; α→1 for strongly velocity-saturated devices). The clip is our notation and does not appear in [1]: its lower bound is the cutoff row of (2) written into the drive, and its upper bound is a numerical guard for the chain of Section II-G, in which a stage output may overshoot its rail by a few percent for a few time steps. In the paper's own setting the gate never exceeds the supply and the upper bound is never reached.

Equation (2) has no single-expression form in [1]; the one used here is ours, and it follows from factoring I'D0 out of the triode row. The two conducting rows then read

ID=I'D0⋅VDSV'D0  (VDS<V'D0), ID=I'D0⋅1  (VDS≥V'D0)

In the first row the ratio VDS/V'D0 is below 1 and in the second it is 1 or above, so in each row the multiplier of I'D0 is whichever of 1 and VDS/V'D0 is the smaller. The two rows meet at VDS=V'D0, where both give I'D0, and the current is continuous. The conducting part of (2) is therefore

ID=I′D0 min1, VDSV′D0    6

Substituting (3) and (4) into (6),

ID=ID0 Dα min1, VDSVD0 Dα/2    7

Equation (7) is the device model in the form used below: a full-drive current, a gate factor Dα, and a drain factor that saturates at unity.

The device model (2)–(5) was checked against the transistors actually used in the predrivers of two test buffers, simulated in HSPICE from their model cards at their drawn dimensions. VTH and α are extracted as in [1, App. A]: VTH is the value for which the saturation current is a straight line against VGS−VTH on logarithmic axes, and α is the slope of that line. Reference [1] gives no procedure for VD0; it is taken here as the breakpoint that best fits the piecewise form (6) to the output curve at full gate drive. Fig. 2 shows the result and Table III lists the two normalized parameters. The measured α is 1.10–1.63, between the short-channel limit of 1 and the long-channel value of 2 [2], and the saturation voltage is 0.34–0.57 of the supply, comparable to the 0.5–0.55 used in [1].

Fig. 2. HSPICE characteristics of the predriver devices of test buffers ex2 and inv_chain, at their drawn dimensions. (a) Output curves at full gate drive, normalized; the legend gives the fitted x0=VD0/VDD. (b) Output curves of one device at five gate drives (solid) against the α-power law (2) with the extracted α and VD0 (dashed; dots mark the saturation voltage V'D0 of (4)). (c) Saturation current against gate drive; the slope is α.

Table III: Device parameters of (2)–(5) extracted in HSPICE from the predriver transistors

| Device | Model card | Drawn length | VDD​ | x0=VD0/VDD​ | α​ |
| --- | --- | --- | --- | --- | --- |
| ex2 predriver NMOS | hspice.mod (BSIM3v3) | 600 nm | 3.3 V | 0.472 | 1.10 |
| ex2 predriver PMOS | hspice.mod (BSIM3v3) | 600 nm | 3.3 V | 0.559 | 1.63 |
| inv_chain NMOS | HL18G-S3.7S | 180 nm | 1.8 V | 0.343 | 1.28 |
| inv_chain PMOS | HL18G-S3.7S | 180 nm | 1.8 V | 0.569 | 1.41 |

## C. Normalization

Let the stage output move from its resting rail toward its destination rail, a distance VDD. Define, for a rising output,

v≜VoVDD,  u≜VDD−VinVDD    8

so that v is the output’s progress toward its destination and u is the input’s progress toward the level that fully turns on the working (pull-up) device: for a rising output the pull-up PMOS does the work, and it is driven harder as the input falls, so u counts how far the input has come down from the supply. Both run from 0 to 1 during a transition. Define the normalized device constants

vt≜VTHVDD,  x0≜VD0VDD    9

A note on vt, because the symbol changes meaning. In (9) it is the device threshold as a fraction of the supply, 0.14–0.33 for the devices of Table III. From Section II-J onward it is the threshold of a modeled stage, fitted to the file and then calibrated, and its fitted values (0.03–0.70 across the test buffers) are not device thresholds. The same symbol is kept because it plays the same role in (19).

With the PMOS source at VDD and the NMOS source at ground, the four terminal voltages follow from (8):

VGS,P=u VDD,  VDS,P=1−v VDD,  VGS,N=1−u VDD,  VDS,N=v VDD    10

Substituting (9) and (10) into (5), VDD cancels and the gate drives of the two devices become

DP=gu,  DN=g1−u,  gx≜clipx−vt1−vt, 0, 1    11

Substituting (9) and (10) into the drain ratios of (7),

VDS,PVD0=1−vx0,  VDS,NVD0=vx0    12

Applying (7) to each device with (11) and (12):

IP=ID0,P guα min1, 1−vx0 guα/2    13

IN=ID0,N g1−uα min1, vx0 g1−uα/2    14

Equations (13)–(14) use one vt, α and x0 for both devices; this is the same symmetric-device simplification adopted in [1, Sec. VI], where the two indices are averaged when they differ.

## D. Normalized stage dynamics

From (8), dVo/dt=VDD dv/dt. Substituting this and (13)–(14) into (1) and dividing by C VDD:

dvdt=sup guα min1, 1−vx0 guα/2−sdn g1−uα min1, vx0 g1−uα/2    15

where

sup≜ID0,PC VDD,  sdn≜ID0,NC VDD    16

Equation (15) is exact with respect to (1)–(5). Its parameters are dimensionless except sup and sdn, which have units of swings per second; 1/sup is the time the stage would take to cross the full swing at its full-drive current. At u=1, (15) reduces to a constant-slope ramp for 1−v>x0 followed by an exponential approach with time constant x0/sup=C VD0/ID0,P, which is the CLR3 of [1, App. B].

Note that α appears twice in each term of (15): in the gate factor gα and in the saturation boundary x0 gα/2.

## E. Reduction to the stage law

Two approximations reduce (15) to the form used in this work.

A1 (gate exponent). In (15) the gate factor is g(u)α with the measured device exponent α, 1.10–1.63 for the devices of Table III. The model keeps the form but treats the exponent as a model parameter p:

h(x)≜g(x)p=clipx−vt1−vt, 0, 1p  (17)

The symbol h marks nothing more than that the exponent is no longer the device's. A full-swing waveform cannot set p: the stage input dwells near g=1, where gp=1 for every p, and only the brief passage through intermediate drives depends on it (Section II-K). In this work p=1 is adopted, so (17) reduces to h(x)=g(x). That is the approximation: A1 drops the measured exponent and makes the drive current linear in the gate above threshold, the short-channel limit of [1]. The parameter p is kept in the formula only so that the exponent can be restored if a measurement supplies it.

A2 (fixed saturation boundary). In (15) the boundary between the constant-current ramp and the resistive tail is x0gα/2: it equals x0 at full drive and shrinks when the gate is only partly on. The model replaces it by a constant that does not depend on the gate:

r(x)≜min1, xxlin  (18)

For a known device the natural value is the full-drive boundary, xlin=x0, and at g=1 (18) then coincides with the boundary in (15); the two differ only under partial drive. The model, however, never knows the device, so the constant is given its own name: xlin is an effective number, set to 0.45 in this work for the reasons given after Fig. 3, and the measured x0 of Table III (0.34–0.57) is only a check on it.

Applying (17) and (18) to (15) yields the stage law:

dvdt=sup hu r1−v−sdn h1−u rv    19

Comparing (19) with (15) term by term: sup and sdn are unchanged; hu replaces guα (A1); r⋅ replaces the drive-dependent drain factor (A2). The second appearance of the gate drive in (15), inside the boundary, is the only structural element removed.

Fig. 3. The stage law (19). (a) Gate factor h(u) of (17) for three exponents. (b) Drain factor of the charging term: the fixed boundary r(1−v) of (18), against the drive-dependent boundary of (15) at three gate drives (α=1, x0=0.45); the two coincide at full drive. (c) Response of one stage to an input step: a constant-slope ramp while 1−v>xlin, then an exponential approach to the rail.

The value used is xlin=0.45. It is a built-in constant of the method, not a quantity read from the IBIS file. It was chosen during development from fits of (19) to the individual probed predriver stages of two test buffers, which gave 0.37–0.47 on ex2 and 0.52–0.63 on inv_chain. Later evidence agrees with it: fitted to the probed gate with xlin free, the chain gives 0.41 on ex2 and 0.48 on inv_chain; the measured x0 of Table III is 0.34–0.57; and [1] uses 0.5–0.55. In the pad-level tests of Section II-N it is fixed at 0.45 on ten of the twelve buffers; on inv_chain and io_buf it was instead fitted to the file together with the other stage parameters. Section II-K shows why the file constrains it only weakly. The default p=1 is likewise a built-in constant; it is the lower limit of the measured α (1.10–1.63).

As a check that (19) is the model of [1] and not merely similar to it, one stage with p=α was driven by a linear input ramp and its delay compared with the closed-form delay of [1, eq. (5)]. The two agree within 4 ps for input ramps of 0.05 to 0.8 ns (stage delays of 0.24 to 0.35 ns), with the opposing device either present or switched off; they separate only for ramps slower than the range of validity stated in [1].

For a falling output, (8) becomes v=VDD−Vo/VDD and u=Vin/VDD; (10) exchanges the P and N rows, and (19) is unchanged.

## F. From the stage law to a buffer model: the road map

Sections II-A to II-E ended with (19), the law of one predriver stage. A buffer is more than one stage: K of them in a row, then the output transistor whose gate the last stage drives, then the die capacitance, the package and the pad. This section lays out the pieces of the complete model, the order in which their numbers are found, and the example that runs through the rest of Section II. Each later section then takes one piece.

### 1) The pieces

Fig. 4 shows the signal path. From left to right: a comparator turns the input pin into a 0-or-1 command; K identical stages obeying (19) pass that command along, each adding delay and rounding the edge, and the output of the last stage is the gate of the output pull-up transistor on a 0-to-1 scale; a static map M turns the gate level into the conducting fraction Ku, and its complement into Kd; and the fractions scale the I–V tables of the file to give the current into the die node, which Ccomp and the package turn into the pad voltage. Everything from the fractions onward is the existing converter's and is unchanged. The proposed method replaces only how Ku(t) and Kd(t) are generated: by a gate that moves, instead of a table replayed against the clock.

Fig. 4. Signal path of the model. The blocks added by the proposed method replace the way the conducting fractions Ku(t) and Kd(t) are generated; the I–V tables, Ccomp and the package elements are those of the IBIS file and are unchanged. Numbers in parentheses are equation numbers.

### 2) What is known, what is assumed, what is fitted

The pieces carry four kinds of numbers (Table V). The I–V tables, Ccomp, the package and the full-swing Ku(t), Kd(t) come from the file and are never changed. Two constants of the stage law, xlin and p, and the three candidate shapes of the map are built into the method; their values were chosen during development with knowledge of the test buffers' transistors, and they are the method's assumptions. The stage rates sup, sdn and a first value of the threshold vt are fitted to the file. The stage count K, the map shape and the final vt are settled by one measurement.

Table V: Inputs of the model and their origin

| Input | Origin | Fixed when |
| --- | --- | --- |
| I–V tables, Ccomp, package elements, full-swing Ku(t) and Kd(t) | the IBIS file | step 0 |
| xlin = 0.45, p = 1, the three candidate map shapes, the band rule (27) | built-in constants, chosen during development on the twelve test buffers with transistor-level information | before any buffer is seen |
| sup, sdn, a first vt​ | fitted to the file | step 1 |
| stage count K, map shape, final vt | one stressed pad measurement | steps 3 and 4 |

Applying the method to a new buffer therefore needs no transistor-level information, but the method is not derived from the IBIS file alone: it uses one measurement, and four built-in constants whose values were chosen with knowledge of the test buffers' transistors. Whether those constants suit buffers outside the test set has not been tested.

### 3) The procedure, step by step

Fig. 5 draws the procedure and Table IV lists it. The steps are numbered 0 to 4, and that numbering is used throughout.

Step 0, the target (Section II-I). The existing converter solves the file's V–T waveforms for Ku(t) and Kd(t) over one complete transition. Normalised from the settled off level to the settled on level, this curve is the target: the only thing the stage law is ever fitted to.

Step 1, the fit (Section II-J). For an assumed stage count and an assumed map shape, the two rates and the threshold are adjusted until the model's Ku(t), produced by running the chain and reading its gate through the map, lies on the target. The fit is repeated for every candidate stage count and shape.

Step 2, the candidates (Section II-K). The fit cannot choose the stage count or the map shape: several of each reproduce the file equally well. The three stage counts whose residual is within 25 % of the best are kept, and with the three map shapes they make nine candidate models.

Step 3, calibration (Section II-L). One pad waveform is measured for an input pulse too short to complete. For each candidate the threshold is bisected until the model's pad peak equals the measured one.

Step 4, selection (Section II-L). The candidate whose whole pad waveform is closest to the measured one is kept. Section II-M walks ex2 through all five steps with its numbers, and Section II-N verifies the result at the gate and at the pad on twelve buffers.

Fig. 5. The procedure as a flow. Grey: taken from the IBIS file. Orange: assumed, built into the method. Teal: fitted or computed. Red: the one measurement. Numbers in parentheses are equation numbers.

Table IV: The procedure. p=1 is fixed; xlin=0.45 is fixed on ten of the twelve test buffers and fitted in step 1 on the other two.

| Step | What is determined | Data used | Eq. |
| --- | --- | --- | --- |
| 0 | I–V tables, Ccomp, full-swing Ku(t) and Kd(t); the normalised target | IBIS file, existing converter | (24), (25) |
| 1 | sup, sdn and an initial vt, for each candidate stage count and map shape | full-swing Ku(t) of the file | (26) |
| 2 | Candidate set: three stage counts × three map shapes | fit residuals of step 1 | (27) |
| 3 | vt, for each candidate | peak of one stressed pad waveform | (28) |
| 4 | Stage count K and map shape | the same stressed pad waveform, whole shape | (29) |

### 4) The running example

ex2 is the running example: a 3.3 V buffer with a three-inverter predriver, whose transistor netlist is available so that every internal quantity can be checked. Two settings of it appear. The figures that explain the chain, the map and the fit (Fig. 6 to Fig. 16) use the map shape and Ccomp measured on its netlist, (0.57, 0.64) and 1.7 pF, so that the model's gate can be compared with the probed gate. The procedure itself, in Sections II-L to II-N, uses only what the file and one measurement provide: Ccomp estimated from the file (2.64 pF, Section II-M) and the three candidate shapes. The two settings give different fitted numbers for the same buffer, and the text says which is in use.

## G. The chain: from the input pin to the output gate

The predriver is modeled as K stages, each obeying (19) with one shared parameter set {sup,sdn,vt,xlin,p} (approximation A3, identical normalized stages). Because (8) expresses every node as its own progress from 0 to 1, an inverting stage needs no sign: each stage's output is simply the next stage's input. The first stage is driven by a comparator on the input pin at half the supply:

u1(t)=1 if Vin(t)>VDD/2, else 0; uk+1(t)=vk(t), k=1,…,K−1  (20)

The output of the last stage is the gate of the output pull-up device, normalized from off to on, and the pull-down gate is taken as its complement:

GUP(t)≜vK(t), GDN(t)≜1−GUP(t)  (21)

The complement in (21) corresponds to an output stage whose two devices are driven from one predriver node, which is the case for ex2 and inv_chain and their variants. Where the netlist has separate pull-up and pull-down predriver paths, as in io_buf, a second chain with its own stage count and parameters is fitted for GDN.

In words. A stage is a bucket. It fills at the rate sup once the bucket before it is more than vt full, and drains at the rate sdn once that bucket has emptied below 1−vt; within xlin of the rim or of the floor the flow tapers off, as the resistive tail of (19) prescribes. A step at the input starts filling the first bucket at once; the second waits until the first passes vt, and the third waits for the second. The delay of the chain is spent in these handovers, and the shape of its last edge is set by the rate. Fig. 13(b) shows the three fitted stages of ex2: the handovers come at 0.24 and 0.66 ns, the gate reaches the map's threshold at 1.11 ns, and the file's own Ku(t) leaves zero at 1.07 ns.

Fig. 6 shows what the chain does. For a full transition every stage completes its swing and the chain contributes only a delay and an edge shape. For a pulse shorter than the chain's own delay, the input reverses while the later stages are still rising: each stage turns back before reaching its rail, and the gate reaches only part of its swing (0.74 in Fig. 6(b)). Because a stage delivers no current until its input passes vt, a pulse loses amplitude at every stage and, once its peak falls below vt, it is not passed on at all. This is why the stage count is a first-order parameter of the model rather than a refinement. The abrupt cutoff is a property of the hard threshold in (17); in a transistor chain the loss is gradual.

Fig. 6. One chain, two inputs (ex2: K=3, sup=2.19/ns, sdn=2.12/ns, vt=0.487, xlin=0.45). Top: the stage nodes of (19)–(21). Bottom: the conducting fraction obtained from the gate through the map (22), normalized. (a) Full transition. (b) An 810 ps input pulse: the gate turns back at 0.74 of its swing and Ku reaches 0.56.

For the fits of Sections II-J and II-N, (19)–(21) are integrated with Heun's method (second-order Runge–Kutta) at 1 ps steps, each state limited to [−0.05,1.05]. Explicit Euler at 2 ps was found to be inadequate for the fastest stages (time constants near 20 ps): it passed a 119 ps pulse that the same chain extinguished in ngspice, whereas Heun agrees with ngspice. In the circuit model each stage is a behavioral current source charging an explicit capacitor Cg, so that vk is a circuit node:

Cgdvkdt=Cg⋅109sup h(vk−1) r(1−vk)−sdn h(1−vk−1) r(vk)

with the rates in swings per nanosecond; Cg cancels and its value is immaterial.

## H. The map: from the gate to the conducting fraction and the pad

The stage law produces the gate trajectory GUP(t) and nothing else; it makes no statement about the output current. An IBIS-based driver model, on the other hand, needs at each instant the fraction Ku of the pull-up I–V table that is conducting, and the fraction Kd of the pull-down table. A second relation, separate from the stage law, connects the two. It is called the map here.

### 1) What the map is

The pull-up table of the file is the current of the output transistor against pad voltage with its gate fully on. With the gate only partly on, the transistor delivers less, and the reduction is taken to be one multiplier at every pad voltage, which is the assumption any Ku-based IBIS model already makes:

I(Vpad,g)=Ku(g) IPU(Vpad)

Here g is the gate of the output transistor on a scale from 0 (device off) to 1 (device fully on); in the model, g=GUP, the output of the last stage. The map is the function Ku(g): how much of the table conducts when the gate is at level g. It is a curve against the gate, not against time.

Where the gate node can be probed, this curve can be measured, and no formula is needed. At each instant of a full transition the gate level and the file's Ku are read together (Fig. 7(a)); plotting one against the other gives Ku as a function of the gate (Fig. 7(b)). On the three buffers where this was done the curve traced while the gate rises coincides with the one traced while it falls, within 0.07–0.11, so Ku does depend on the gate alone, without memory.

Fig. 7. The map. (a) With the gate node of ex2 probed, the gate level (squares) and the file's Ku (circles) are read at the same instants of a full transition. (b) Plotting Ku against the gate gives the measured map; the dashed line is the formula (22) fitted to it, with (vt,map,a)=(0.57,0.64). (c) The three shapes offered by the method when the gate cannot be probed, with the measured map of ex2 for reference.

### 2) The formula

The measured curve is described by a formula with two shape parameters, scaled between the settled off and on levels of the file's own Ku and Kd:

Ku(t)=koff+(kon−koff) M(GUP(t)), M(g)≜clipg−vt,mapgs−vt,map,0,1a  (22)

Kd(t)=doff+(don−doff) M(GDN(t))  (23)

Table VI: The symbols of the map (22)–(23)

| Symbol | Meaning | Values |
| --- | --- | --- |
| g​ | Gate of the output transistor, 0 (off) to 1 (fully on); the output of the last stage | a waveform |
| vt,map​ | Gate level at which M leaves zero | 0.4 or 0.5 in the candidates; 0.49–0.57 measured |
| a​ | Exponent that bends the curve: 1 is a straight line, below 1 rises steeply and then flattens | 0.6, 0.7 or 0.9 in the candidates; 0.60–0.78 measured |
| gs​ | Gate level at which M reaches 1 | 1 in all results reported here |
| koff,kon,doff,don​ | Settled levels of the file's Ku and Kd | close to 0 and 1 |

Read from the inside out, M does three things. Subtracting vt,map sets where the curve starts: below that gate level nothing conducts. Dividing by gs−vt,map rescales the remainder so that it runs from 0 at the start to 1 at g=gs; the clip keeps it there. The exponent a bends the curve between those two ends. With gs=1 two numbers therefore fix the whole curve.

The form is borrowed from the gate factor (5) of a transistor, a threshold followed by a power of the drive, because the map describes how the output transistor turns on. Its parameters are nevertheless empirical. Fitted to the measured maps they are (0.57, 0.64) on ex2, (0.49, 0.60) on inv_chain and (0.50, 0.78) on io_buf; they are not identified with the threshold voltage or the index α of the output device, and no derivation of their values is offered. They are also distinct from the stage parameters vt and p of (17), which describe the predriver. The formula follows the main rise of the measured map of ex2 within about 0.06 and does not reproduce its slow foot between g=0.4 and 0.57 (Fig. 7(b)).

### 3) Why the file does not contain the map

The file gives Ku against time. The map is Ku against the gate. Converting one into the other requires the gate against time, as in Fig. 7(a), and that is the one waveform an IBIS file does not have. The shape of the file's Ku(t) is the shape of the map stretched by how fast the gate moved, and the two cannot be separated without the gate.

The method therefore does not obtain the map from the file. It offers three fixed shapes (Fig. 7(c)) and leaves the choice among them to the stressed measurement (Section II-L). The three are built-in constants (Table V): (0.50, 0.70) lies near the middle of the three measured maps; (0.40, 0.60) and (0.40, 0.90) were retained from an earlier search scored on the same twelve test buffers. Their generality is untested, and this is the least well founded element of the method.

### 4) From the gate waveform to the conducting fraction

With a map chosen, the model's Ku(t) is obtained by evaluating the two relations one after the other: (19)–(21) give GUP(t), and (22) is applied to each of its values. Table VII does this for a single stage after an input step; Fig. 8 does it for the fitted three-stage chain of ex2.

Table VII: Worked example: one stage (sup=2 per ns, xlin=0.45) read through the map (vt,map=0.57, a=0.64)

| Time after the input step (ns) | Gate g, from the stage law (19) | M(g), from the map (22) |
| --- | --- | --- |
| 0.10 | 0.20 | 0 (gate below vt,map) |
| 0.20 | 0.40 | 0 (gate below vt,map) |
| 0.30 | 0.60 | 0.19 |
| 0.40 | 0.74 | 0.56 |
| 0.60 | 0.89 | 0.84 |
| 1.00 | 0.98 | 0.97 |

The gate rises as a ramp of 2 per ns until it is within xlin of its rail and then approaches it exponentially. Nothing conducts until the gate passes 0.57, 0.29 ns after the step; the conducting fraction then rises quickly, because the map is steep just above its threshold.

Fig. 8. Construction of the conducting fraction on ex2 (K=3, map shape (0.57, 0.64)). Left: the gate trajectory given by the stage law. Middle: the map (22); the dots are the gate values of the left panel at six instants. Right: the resulting Ku(t), normalized, with the same instants marked. (a)–(c) Full transition, with the parameters of the fit (26); in (c) the model is compared with the Ku(t) of the IBIS file, and this comparison is the fit of Section II-J. (d)–(f) An 810 ps input pulse, after calibration: the gate turns back at 0.74, the map is read only up to that level, and Ku reaches 0.56.

The top row of Fig. 8 is the situation the file records. The gate sweeps its full range, the whole map is read, and the result in (c) can be compared with the file. The bottom row is the situation the file does not record. The input is cut short, the gate turns back before reaching its rail, and only the lower part of the map is read. The conducting fraction in (f) rises to 0.56 and returns to zero: a waveform that no table of the file contains, produced by the same two relations with no additional parameter.

### 5) From the conducting fraction to the pad

The current delivered to the die node is that of the existing converter:

Idie(t)=Ku(t) IPU(Vdie)+Kd(t) IPD(Vdie)+IPC(Vdie)+IGC(Vdie)  (24)

where IPU and IPD are the pull-up and pull-down I–V tables of the file, held at their end values outside the tabulated range, and IPC, IGC are the power- and ground-clamp tables, present only when the file contains them (io_buf does; ex2 and inv_chain do not). Ccomp loads the die node and the package R, L, C of the file connect it to the pad. Equation (24) was checked against the generated subcircuits and is unchanged by the proposed method, which replaces only the source of Ku(t) and Kd(t); the converter's short first-order lag on the fractions (5 ps) and its residual correction term are also retained as they are.

## I. Step 0: the target, the file's own Ku(t)

The fit of the next section needs a curve to fit to, and nothing else in the method is fitted to anything. That curve is the file's own conducting fraction through one complete transition. This section shows where it comes from, on ex2 (Fig. 9).

What the file holds. ex2's file tabulates the pull-up and the pull-down current at 100 pad voltages each (Fig. 9(a)), and the pad voltage against time for a rising and for a falling transition into two fixtures, 50 Ω to 0 V and 50 Ω to 3.3 V, 1000 samples over 8 ns each (Fig. 9(b)). The two fixtures are the point: the same transition recorded into two different loads.

How the converter solves for the two fractions. At each instant of the rising transition the current into the die node must balance in both fixtures. With the pull-up table scaled by Ku and the pull-down table by Kd, and Vi(t) the pad voltage recorded in fixture i,

Ku(t) IPU(Vi(t))+Kd(t) IPD(Vi(t))=Vfix,i−Vi(t)Rfix−CcompdVidt, i=1,2

(the clamp currents join the right-hand side when the file has them). Two equations, two unknowns: the converter solves the pair at every sample and obtains Ku(t) and Kd(t) for the rising transition, and in the same way for the falling one (Fig. 9(c)). This is the standard two-waveform solution; it is not part of the proposed method, which takes its result as given. Note what the solution contains: Ku against time, for the one transition the file recorded, including the buffer's own delay.

From the solution to the target. The existing converter builds its circuit model so that, driven through one complete transition, it reproduces this solution; the model's internal node that carries the table value, before the converter's small residual correction, follows the solved Ku(t) within 0.015 rms on ex2 once the two are aligned on the input edge (Fig. 9(c)). The target is that node, recorded in an ngspice run of the existing model through one 10 ns input pulse and normalised between its settled off level at trest and its settled on level at ton:

Ku(t)=clipKu(t)−Ku(trest)Ku(ton)−Ku(trest),0,1, trest=4.5 ns, ton=12 ns  (25)

(the input edge is at 5 ns). Fig. 9(d) shows the result, sampled every 2 ps from 4 to 21 ns, together with the input that the model's chain receives during the fit: an ideal 10 ns pulse starting at the mid-point of the input edge, (20). Its rising edge constrains sup and its falling edge sdn. Nothing in this curve says how many stages produced it or where their thresholds lie; that is the subject of Section II-K.

Fig. 9. The target, on ex2 (file-only run at the measured Ccomp of 1.7 pF). (a) The I–V tables of the file. (b) Its V–T waveforms: one rising and one falling transition, each recorded into two fixtures. (c) Ku(t) and Kd(t) solved from (b) by the existing converter (grey), and the conducting fractions of the existing circuit model through a 10 ns input pulse (colour); the solution is aligned on the input edge. (d) The normalised target (25) and the chain input (20) used in the fit.

## J. Step 1: fitting the stage law through an assumed map

This is the step that gives the stage law its numbers, and it deserves to be seen slowly. The question, the score, what each number does, how the minimum is found, why several stages are needed, and what the fit leaves open are taken in turn, on ex2 with its measured map shape and Ccomp (Section II-F.4).

### 1) The question

Table VIII sorts the quantities of the model by their status in this step. Three numbers are unknown; everything else is given by the file, assumed for the candidate, or built in. The order matters: the map is assumed first and its two numbers frozen (Section II-H.3), and only then are the stage numbers adjusted. The map is an input to the fit, not a result of it.

Table VIII: The quantities of the model and their status in step 1

| Quantity | Status in step 1 | In the ex2 example |
| --- | --- | --- |
| target Ku(t) | given: the file, through the existing converter (Section II-I) | Fig. 9(d) |
| map shape (vt,map,a) | assumed for the candidate; three shapes offered | (0.57, 0.64) here; (0.5, 0.7), (0.4, 0.6), (0.4, 0.9) in the procedure |
| stage count K | assumed for the candidate; three counts from the band (27) | 3 here; 3, 4, 5 in the procedure |
| xlin, p​ | built in | 0.45, 1 |
| sup, sdn​ | fitted | 2.19 and 2.12 per ns |
| vt​ | fitted here; replaced by the calibration of step 3 (Section II-L) | 0.528 here; 0.487 after calibration |

For a given stage count K and the assumed map shape, the stage parameters minimize the root-mean-square mismatch between the model's conducting fraction and the target:

{sup,sdn,vt}⋆=argmin rmstM(vK(t))−Kufile(t), xlin, p fixed  (26)

with 0≤vt≤0.7, over 4–21 ns sampled every 2 ps. For a buffer with a separate pull-down path, the pull-down chain is fitted in the same way to Kd(t) through its own map; fitting it through the Ku target leaves its onset unconstrained, because that target is zero wherever Kd turns on (an onset 250 ps early was observed on an open-drain test buffer).

### 2) The score: what the residual measures

For a trial set of the three numbers, the model's Ku(t) is produced exactly as in Fig. 8: the chain is run from the 10 ns input pulse, and the gate is read through the map at every sample. At each of the 8500 samples the model is subtracted from the target; the differences are squared, averaged, and the square root is taken. That one number is the residual of (26). Fig. 10 shows it for a poor trial and for the fit: the shaded gap in (a) and (b) is what is being measured, and (c) shows the difference over the whole window. The residual is small only when the gap is small everywhere. Because the target is flat at 0 or 1 for almost all of the 17 ns, the residual is in practice accumulated within the two edges, each about 0.7 ns wide on ex2: a residual of 0.023 means that, averaged over the window, the model is 2.3 % of the full swing away from the file, and most of that is a small timing and shape difference at the edges.

Fig. 10. What the residual of (26) measures, on ex2 (K=3, map shape (0.57, 0.64), xlin=0.45). (a) A poor trial: the shaded gap between the model and the file gives a residual of 0.296. (b) The fit: residual 0.023. (c) The difference model − file over the whole fit window for both; the residual is its root-mean-square, and it lives at the two edges.

### 3) What each number does

Fig. 11(a)–(c) varies one fitted number at a time around the fitted values. sup is the charging rate of every stage: a larger value brings the rising edge earlier and makes it steeper, a smaller one delays and flattens it. sdn does the same for the falling edge. vt is the level a stage's input must reach before the stage starts: at fixed rates it moves the edge later or earlier without changing its steepness, because the delay between stages is the time each stage's output takes to reach vt. The three numbers therefore have distinct, visible jobs: two slopes and one delay.

Fig. 11(d) builds a fit one number at a time from a poor first guess, which the real search does not do but which shows what each number buys: from a first guess (residual 0.296) adjusting sup fixes the rising edge (0.201), adjusting sdn fixes the falling edge (0.060), and adjusting vt places the edges in time (0.023).

Fig. 11. The fit (26) seen as three knobs, on ex2 (K=3, map shape (0.57, 0.64), xlin=0.45). (a)–(c) The model's Ku(t) as sup, sdn and vt are varied one at a time around the fitted values, against the file's Ku(t); (b) shows the falling edge at 10 ns. (d) A fit built one number at a time from a poor first guess; the real minimization moves all three together.

### 4) How the minimum is found

The three numbers are found by Nelder–Mead minimization, chosen because the residual is not differentiable: the chain is a simulation with thresholds and clips. The method keeps a simplex of four trial points in the three-dimensional parameter space. At each iteration it evaluates the residual at the point reflected through the far side of the simplex from its worst corner; if that is better than the best corner it tries a further step in the same direction, if it is worse than the second-worst corner it tries a point halfway back, and if nothing helps it shrinks the whole simplex towards its best corner. The simplex therefore walks downhill and contracts around a minimum. The rates are searched on a logarithmic scale, so that 1 and 30 per ns are equally reachable; vt is searched linearly and confined to [0,0.7] by returning a large residual outside it. The search stops when the simplex has collapsed (corners within 0.001 of each other and residuals within 0.00001) or after 800 iterations. It is run from three starting points, sup=sdn=2, 8 and 30 per ns with vt=0.4, and the best end point is kept.

Fig. 12 shows the search on ex2. Panels (a) and (b) are the residual over two planes through the fitted point. The valley is narrow in the rates: at the fitted vt, a change of 27 % in sup in either direction triples the residual. Along vt it is shallow on the low side, 0.128 at vt=0 falling gently to 0.026 at 0.54, and steep above, 0.091 at the 0.7 bound. The three runs (c) took 100, 132 and 142 evaluations of (26), each a 17 ns chain simulation, before their simplex collapsed. Only the run from 2 per ns reached the minimum (0.0233); the run from 8 per ns stopped at 0.066 with vt on its bound, and the run from 30 per ns at 0.22 with sdn at 52 per ns, both in side valleys of the landscape. The search is local, and this is why one start is not enough.

Fig. 12. The search, on ex2 (K=3, map shape (0.57, 0.64), xlin=0.45). (a) The residual of (26) over sup and vt at the fitted sdn, and (b) over sup and sdn at the fitted vt; darker is lower. Squares: the three starting points; stars: where each run ended; the red circle: the fit. The paths are the best point found so far, projected onto each plane. (c) The best residual against the number of evaluations for the three runs.

### 5) Why several stages

With a single stage, one number sets both the delay and the edge: Ku starts when the gate passes vt,map, at about vt,map/sup after the input, and the edge takes about (1−vt,map)/sup. On ex2 the file's Ku(t) starts 1.1 ns after the input edge and rises in 0.66 ns, which no single value of sup can give: the best one-stage fit (Fig. 13(a)) is early and slow, with a residual of 0.083 against 0.023 for three stages. With several identical stages the delay is spent in the handovers (Fig. 13(b)) and the edge belongs to the last stage, so delay and edge are set separately. This is the reason for the chain, and it is also why the stage count cannot be read from the fit: beyond the minimum number, more stages redistribute the same delay (Section II-K).

Fig. 13. Why a chain, on ex2. (a) The file's Ku(t) against the best single-stage fit (sup = 0.64/ns, vt = 0.14) and the best three-stage fit (sup = 2.19/ns, vt = 0.528). (b) The three stage outputs of the fitted chain: each stage starts when the one before it passes vt (dashed), and Ku leaves zero when the gate passes vt,map (dotted); the times are marked.

### 6) The result

On ex2 with K=3, map shape (0.57, 0.64) and xlin=0.45, (26) gives sup=2.190/ns, sdn=2.124/ns and vt=0.528 at a residual of 0.023 (Fig. 14(a)); on inv_chain with K=7 and its measured shape (0.49, 0.60) the residual is 0.008 (Fig. 14(b)). Three numbers are fitted, to a curve that the file already contains; no stressed data is used in this step. The figure also shows the one curve the file does not contain and the fit produces as a by-product: the gate trajectory.

Fig. 14. Result of the fit (26) with xlin=0.45: the file's normalized Ku(t), the fitted model, and the model's gate trajectory, which the file does not contain. (a) ex2, K=3, map shape (0.57, 0.64). (b) inv_chain, K=7, map shape (0.49, 0.60).

Fig. 15 and Table IX give the same fit on all twelve test buffers, now with the file-only settings of the procedure: Ccomp estimated from the file, and the stage count and map shape that the procedure finally selected (Section II-N). The residuals are 0.006–0.033, and on every buffer the model's Ku(t) lies on the file's. Two things in the table deserve notice. First, on 7 of the twelve buffers the fitted vt sits on its upper bound of 0.7: with these map shapes the fit would place the handovers even later than the bound allows, and the calibration of step 3 then lowers vt on all eleven buffers calibrated through the threshold (last column). This is the non-identifiability of the threshold, seen from the other side (Section II-K): the file pushes vt to wherever the assumed map lets the composition match, and the measurement puts it where the short pulse says. Second, io_buf's single-stage chain could not reach the measured peak by its threshold alone and was calibrated through the rate scale instead (28).

Table IX: The fit (26) on the twelve test buffers with the file-only settings, for the stage count and map shape the procedure selected; xlin = 0.45 except inv_chain (0.28) and io_buf (0.41), where it was fitted. The last column is the threshold after step 3.

| Buffer | Ccomp (pF) | K​ | Map shape | sup (/ns) | sdn (/ns) | vt fitted | Residual | vt calibrated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ex2 | 2.64 | 3 | (0.5, 0.7) | 2.55 | 2.69 | 0.700 | 0.0233 | 0.632 |
| ex2_base | 2.49 | 3 | (0.5, 0.7) | 2.54 | 2.70 | 0.700 | 0.0224 | 0.632 |
| ex2_slowpre | 4.65 | 3 | (0.5, 0.7) | 1.63 | 1.84 | 0.427 | 0.0236 | 0.413 |
| ex2_skewp | 1.44 | 3 | (0.5, 0.7) | 2.67 | 2.81 | 0.700 | 0.0329 | 0.604 |
| ex2_weak | 1.27 | 3 | (0.5, 0.7) | 2.88 | 2.84 | 0.699 | 0.0259 | 0.626 |
| ex2_nomiller | 2.3 | 3 | (0.5, 0.7) | 2.54 | 2.70 | 0.699 | 0.0219 | 0.632 |
| inv_chain | 0.66 | 6 | (0.4, 0.9) | 19.34 | 18.97 | 0.700 | 0.0078 | 0.582 |
| inv_base8 | 0.37 | 7 | (0.4, 0.6) | 23.76 | 24.00 | 0.610 | 0.0065 | 0.539 |
| inv_stage4 | 0.43 | 3 | (0.4, 0.9) | 25.61 | 24.67 | 0.700 | 0.0061 | 0.626 |
| inv_skewp | 0.24 | 7 | (0.4, 0.6) | 22.36 | 21.20 | 0.537 | 0.0094 | 0.473 |
| inv_weak | 0.35 | 7 | (0.4, 0.6) | 22.96 | 23.52 | 0.630 | 0.0064 | 0.571 |
| io_buf | 1.68 | 1 | (0.5, 0.7) | 0.40 | 1.99 | 0.699 | 0.0186 | rates × 1.10 |

Fig. 15. The fit (26) on the twelve test buffers with the file-only settings, rising edge: the file's normalized Ku(t) and the model's, for the candidate the procedure selected. The residual in each title is over the whole fit window.

### 7) The fit succeeds whichever map is assumed

A good fit does not show that the assumed map was right. If the map is made to start earlier, the fit delays the gate to compensate, and the composition M(GUP(t)) still lands on the file's curve. Table X gives the residual of (26) on ex2 and inv_chain for six different shapes.

Table X: Residual of the fit (26) for six assumed map shapes (xlin=0.45; K=3 for ex2, 7 for inv_chain)

| Map shape (vt,map,a) | (0.3, 0.6) | (0.4, 0.6) | (0.4, 0.9) | (0.5, 0.7) | (0.6, 0.6) | (0.6, 0.9) |
| --- | --- | --- | --- | --- | --- | --- |
| ex2 | 0.0220 | 0.0227 | 0.0214 | 0.0226 | 0.0238 | 0.0226 |
| inv_chain | 0.0078 | 0.0078 | 0.0078 | 0.0079 | 0.0084 | 0.0082 |

The residual varies by 11 % on ex2 and 8 % on inv_chain across the six shapes, and its small preference does not point at the measured map: on ex2 the shape nearest the measured one, (0.6, 0.6), has the largest residual. The file cannot choose the map.

Fig. 16. Two assumed maps on ex2 (K=3), each with the stage law fitted through it to the same file. (a) The two maps. (b) The gate trajectory the fit produces for each: the earlier map is paired with a later gate. (c) Full transition: both reproduce the file's Ku(t). (d) The same two models driven by an 810 ps pulse, before any calibration: one passes the pulse, the other extinguishes it.

Fig. 16 shows what this means. Through the later-starting map (0.6, 0.6) the fit gives vt=0.479; through the earlier-starting map (0.4, 0.6) it gives vt=0.678, a gate that leaves later. For a full transition the two models are indistinguishable (residuals 0.0238 and 0.0227). Driven by an 810 ps pulse they are not: the first delivers a conducting fraction of 0.51, the second none at all, because its higher stage threshold extinguishes the pulse inside the chain. Two models that the file cannot tell apart give opposite answers for the short pulse. This is why a measurement of a short pulse is required, and Section II-K shows that the same holds for the stage count and the other parameters that act only under partial drive.

### 8) What the fit decides and what it leaves open

The fit fixes the two rates and, for the assumed map and stage count, a threshold. It does not decide the map, the stage count or, as the next section shows, even the threshold in any absolute sense: each of those can be changed and compensated by the others without the file noticing.

## K. What the file cannot determine

In plain terms: a complete transition is two edges. Two edges fix two rates and one delay, and the fit has exactly those three numbers to spend. Anything that acts only when a transition is cut short, how many stages share the delay, where each hands over, how the output transistor turns on in between, leaves no mark on a complete transition and cannot be read from it. This section shows that in numbers.

The fit (26) determines the rates well and the structure poorly. Table XI and Fig. 17 give its residual as a function of the stage count for three of the twelve test buffers, each K fitted independently. The transistor netlists of the test buffers are available, so the true stage count is known.

Table XI: Residual of the fit (26) in the Ku domain against the stage count K

| Buffer | K = 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | Netlist |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ex2 | .0829 | .0292 | .0233 | .0224 | .0215 | .0222 | .0235 | .0255 | .0274 | .0294 | 3 |
| inv_chain | .0468 | .0216 | .0141 | .0108 | .0076 | .0071 | .0070 | .0072 | .0072 | .0074 | 7 |
| io_buf (pull-up) | .0176 | .0264 | .0500 | .0639 | .0724 | .0783 | .0825 | .0858 | .0883 | .0904 | 1 |

Fig. 17. Residual of (26) against stage count for the thirteen predriver chains of the twelve test buffers (io_buf has two). Circles mark the netlist stage count; the thick segments mark the band (27).

Two features are evident. First, K=1 fails on ex2 and inv_chain: a single stage cannot produce the required delay together with the required edge rate, so the full swing does constrain the rate. Second, beyond that the residual is flat: on ex2, K=3 to 6 lie within 0.002 of each other and the minimum is at K=5 although the netlist has three stages; on inv_chain, K=6 to 9 differ in the fourth decimal. The margin between the best and the second-best K is at most 0.0007 on twelve of the thirteen chains. Only io_buf's pull-up, which is a single stage, has a distinct minimum (margin 0.0088). This is the same degeneracy as that of the map in Section II-J.7, in numbers.

The same holds for the other quantities that act only under partial drive. The threshold: the fitted vt of ex2 is 0.029 in one fitting pass (K=6) and 0.528 in another (K=3) that differs in Ccomp and map shape, at residuals of 0.0202 and 0.0233. The exponent: at full swing each stage input dwells at the rail, where (17) does not depend on p; in fits of the individual probed stages every p between 1 and 2 reaches a residual of 0.002–0.007. The saturation boundary: if xlin is left free in (26) it settles at 0.206 on ex2 and 0.135 on inv_chain, less than half the measured x0 of Table III, and replacing (18) by the drive-dependent boundary of (15) changes the residual by about 1 % on ex2 and not at all on inv_chain. The file cannot distinguish these alternatives, which is why xlin and p are normally held fixed (Table V) rather than fitted.

A fit with every stage free is worse than uninformative. On ex2 with K=2 and all eight parameters free, the full-swing residual against the probed gate falls to 0.0046, below the 0.0066 of the identical-stage fit with the correct K=3; it achieves this with one very slow stage and one fast one, and such a chain extinguishes short pulses: it predicts a stressed gate peak of 0.228 where 0.758 is measured. Sharing one parameter set among the stages (A3) removes this solution and leaves K as the single structural unknown.

The procedure therefore keeps a band of stage counts instead of a point estimate:

band=the three smallest K with rms(K)≤1.25 minK' rms(K')  (27)

On the twelve test buffers the band contains the netlist stage count in every case. The tolerance of 25 % and the band width of three were chosen on these same buffers, so this is a property of the rule as tuned and not an independent validation.

## L. Steps 2–4: the single stressed measurement

### 1) Why it is needed

After step 1 there is not one model but several, and all of them reproduce the file. They differ in what the file cannot see: the map shape (Section II-J.7), the stage count and the stage threshold (Section II-K). These are exactly the quantities that decide what happens when the input is cut short, because they set how far the gate has travelled when it turns back and how much of the pulse each stage passes on. The candidates therefore agree on a full transition and disagree on a short pulse, and one measured short pulse can tell them apart.

Fig. 18(b) shows how large the disagreement is. For ex2, the nine candidates of step 2, each fitted to the same file, were driven by the same 810 ps pulse. Their pad peaks range from 0.01 to 1.28 V; the measured peak is 0.77 V. Some overshoot it by more than half, others extinguish the pulse altogether. The file alone gives no means of knowing which is right.

### 2) What is measured

The measurement is the pad voltage for one short input pulse into a known load (Fig. 18(a)). Nothing inside the buffer is observed. In the tests below the pulse is the one that reaches about 50 % of the buffer's settled swing into a 50 Ω, 2 pF load, and the measurement is a transistor-level simulation of the buffer; for a user of the method it would be a bench measurement or a simulation supplied by the vendor.

### 3) How it is used

Candidates (step 2). The band (27) supplies three stage counts and Fig. 7(c) three map shapes. Each of the nine combinations has its own fit (26).

Calibration of the threshold (step 3). The stage threshold vt decides how much of a short pulse survives each stage, and so sets the height of the model's pad pulse; it is also the parameter the file determines worst (Section II-K). For each candidate, with V the measured peak of the pad voltage, define the peak error

e(vt)≜maxtVpadmodel(t;vt)−VV, e(vtcal)=0  (28)

and solve e=0 for vt by bisection on [0,0.7], all other parameters held at their values from (26). Each evaluation is one circuit simulation of the complete model; three bracket the interval and seven bisect it. If the threshold cannot bracket the peak, sup and sdn are scaled together within [0.5,2] instead. The threshold is used, and not the rates, because the rates were fitted to reproduce the full swing and moving them degrades it, whereas vt changes when each stage hands over without changing how fast it runs. After this step every candidate reproduces the measured peak (Fig. 18(c)).

Fig. 19 shows the calibration on ex2's selected candidate, built with the file-only settings. The peak error as a function of vt (a) is not monotonic: it is +40 % at vt=0, rises to +59 % near 0.25, and falls steeply above 0.55 to -97 % at the bound, where the chain extinguishes the pulse. A higher threshold means that each stage passes less of a short pulse to the next, so the pad peak falls as vt rises, and the zero crossing is where the model's peak equals the measured one. The bisection (b) needs only the sign of the error: after the three bracketing runs (vt as fitted, 0 and 0.7) the interval [0,0.7] is halved seven times, each halving one ngspice run of the complete model, and the midpoint of the final interval, 0.632, is kept. Panel (c) shows the pad waveform at four of the iterations: too early and too high at low thresholds, on the measured peak at the end. The fitted threshold of this candidate was on its bound, 0.700, with a peak error of -97 %; calibration moved it to 0.632.

Fig. 19. The calibration (28) on ex2's selected candidate (K=3, map shape (0.5, 0.7), file-only settings), for the 810 ps pulse. (a) The peak error against the stage threshold, from a dense sweep (grey) with the ten iterations of the procedure numbered (orange: the three bracketing runs; teal: the seven bisections). (b) The interval each bisection tested, with the sign it found. (c) The pad waveform at four of the iterations against the measurement; the dashed line is the measured peak.

Selection of the stage count and map shape (step 4). Because calibration has forced the peak to match, the peak can no longer distinguish the candidates. They still differ in the rest of the waveform, in when the pulse arrives and how it returns: in Fig. 18(c) the nine pulses arrive between 0.11 ns early and 0.30 ns late. The candidate is chosen by the waveform error over the pulse and its return,

(K,vt,map,a)⋆=argmin rmst∈[tf−0.3 ns, tf+1.5 ns]Vpadmodel(t)−Vpadmeas(t)  (29)

where tf is the falling edge of the input pulse (the shaded interval of Fig. 18(c)). The window was chosen once and was not tuned. On ex2 the selected candidate (K=3, shape (0.5, 0.7)) arrives 7 ps early.

Fig. 18. Use of the stressed measurement on ex2. (a) The measurement: pad voltage for an 810 ps input pulse. (b) The nine candidates as fitted to the IBIS file, driven by the same pulse, before the measurement is used; the dashed line is the measured peak. (c) After the threshold of each candidate is calibrated on the peak (28): all reach the measured peak and differ elsewhere; the shaded interval is the window of (29), and the selected candidate is highlighted. (d) The selected model at the five pulse widths of Section II-N against the transistor-level reference. Time is measured from the input edge.

### 4) What it does and does not establish

One waveform is thus used for two decisions, read from two different features of it: the threshold from its height, the stage count and map shape from its shape. Selecting on the peak instead of the waveform is markedly worse. Averaged over the twelve buffers, the pad-waveform error at all five stressed widths (the rms of (29) evaluated at each width) is 52.3 mV for the candidates chosen by (29), against 51.7 mV for the best candidate available in each grid and 217.3 mV for the candidates that minimize the worst peak error, which achieve their peak by arriving 155–300 ps late.

The measurement can only choose among the candidates it is offered. If the true map is unlike all three shapes, or the true stage count lies outside the band, it selects the least wrong candidate, not the right one. Fig. 18(d) and Section II-N test the outcome at pulse widths other than the one used here.

## M. The procedure end to end: ex2

This section collects what the procedure did on ex2 with the file-only settings, so that the whole chain of decisions can be read in one place. The input was the IBIS file and one transistor-level pad waveform for the 810 ps pulse; everything else in the two tables was computed from those.

Table XII: ex2 through the procedure, file-only settings

| Step | What was done | Result on ex2 |
| --- | --- | --- |
| 0 | Ccomp. The file declares 5.0 pF. The converter's Ku solution cannot exceed 1 for a correct Ccomp, and with 5.0 pF it overshoots to 1.24; the value at the knee of that overshoot against the assumed Ccomp is taken instead (the file's own estimate). | 2.64 pF (the netlist measurement, not used, gives 1.7 pF) |
| 0 | Target: Ku(t) solved from the V–T waveforms, replayed by the existing model, normalised (25). | as Fig. 9(d), at 2.64 pF |
| 1 | Fit (26) for K = 1 … 10 at the default shape (0.5, 0.7); residuals | .0920 .0360 .0233 .0229 .0212 .0202 .0203 .0210 .0221 .0235 |
| 2 | Band (27): the three smallest K within 25 % of the best residual; × three map shapes | K ∈ {3, 4, 5}; nine candidates (Table XIII) |
| 3 | Measured peak of the 810 ps pad pulse; candidates' peaks before calibration; bisection of vt (28), ten runs each | 0.77 V measured; 0.01–1.28 V before; after calibration all within 2.5 % of the peak |
| 4 | Waveform rms (29) at the calibration width; the smallest wins | 56–400 mV across the nine; selected K = 3, shape (0.5, 0.7), 56 mV, arriving 7 ps early |
| check | Worst peak error of the selected model over the five stressed widths (Section II-N) | 8.3 % |

Table XIII: The nine candidates of ex2 through steps 2–4. Columns 3–6: the fit (26) to the file. Peak before: the pad peak of the candidate as fitted, for the 810 ps pulse (measured 0.77 V). vt cal., peak error and rms (29): after calibration, at the calibration width. Lag: arrival of the model's pulse relative to the measurement. Worst: over the five stressed widths, not available to the procedure.

| K​ | Shape | sup​ | sdn​ | vt fit | Residual (26) | Peak before (V) | vt cal. | Peak err. (%) | rms (29) (mV) | Lag (ps) | Worst 5 widths (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | (0.4, 0.6) | 2.44 | 2.81 | 0.700 | 0.0233 | 0.01 | 0.604 | -0.7 | 132 | -108 | 8.0 |  |
| 4 | (0.4, 0.6) | 2.78 | 3.23 | 0.485 | 0.0227 | 1.12 | 0.577 | +2.3 | 150 | +97 | 3.0 |  |
| 5 | (0.4, 0.6) | 2.96 | 3.45 | 0.265 | 0.0225 | 1.28 | 0.544 | -2.5 | 319 | +257 | 2.5 |  |
| 3 | (0.4, 0.9) | 2.53 | 2.73 | 0.700 | 0.0236 | 0.09 | 0.621 | -0.1 | 80 | -21 | 9.8 |  |
| 4 | (0.4, 0.9) | 3.08 | 3.41 | 0.570 | 0.0219 | 1.19 | 0.675 | +0.6 | 216 | +186 | 3.5 |  |
| 5 | (0.4, 0.9) | 2.86 | 3.06 | 0.098 | 0.0211 | 1.22 | 0.544 | -1.9 | 383 | +300 | 2.8 |  |
| 3 | (0.5, 0.7) | 2.55 | 2.69 | 0.700 | 0.0233 | 0.02 | 0.632 | -1.8 | 56 | -7 | 8.3 | selected |
| 4 | (0.5, 0.7) | 2.72 | 2.89 | 0.407 | 0.0229 | 1.19 | 0.571 | +2.4 | 249 | +216 | 2.4 |  |
| 5 | (0.5, 0.7) | 2.88 | 3.01 | 0.065 | 0.0212 | 1.21 | 0.561 | -0.8 | 400 | +300 | 2.3 |  |

Three things can be read off Table XIII. The fit residuals of the nine candidates lie within 12 % of each other, so the file does not rank them. Their pad peaks before calibration run from 0.01 to 1.28 V against 0.77 V measured, so the short pulse does. And after calibration every candidate matches the peak within 2.5 %, so the peak no longer ranks them and the waveform must: the selected candidate has the smallest rms (29) and arrives closest to the measured pulse; the candidates with the smallest worst-case error over five widths (last column) would have been the K = 4 and 5 builds, which the procedure cannot know and which the selector did not pick, so the selection costs 6 points of peak error on this buffer.

## N. Verification

Two claims are verified separately. The first concerns the stage law itself: fitted to a full transition only, a cascade of (19) predicts how the gate responds to a truncated pulse. The second concerns the complete method: built from the IBIS file and one stressed measurement, the model reproduces the pad voltage of stressed pulses.

### 1) Gate level: the stage law extrapolates from full swing to truncated pulses

This test uses the internal gate node of the transistor netlist (node n4 of ex2, vout7 of inv_chain) and therefore is not available to a user of the method; its purpose is to test (19). K identical stages are fitted in the gate domain to the probed full-swing gate; the chain is then driven by the stressed input and compared with the probed stressed gate at five pulse widths, which correspond to pad amplitudes of about 50, 60, 70, 80 and 90 % of the settled swing. No stressed data enters the fit. The comparison is linear superposition of the node's own full-swing rise and fall responses, which is what any linear model of the predriver (an RC cascade, a delay followed by an RC) reduces to.

Table XIV: Peak of the stressed gate, ex2 (fitted at full swing only)

| Pulse width (ps) | 810 | 830 | 858 | 895 | 975 |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.758 | 0.797 | 0.838 | 0.880 | 0.936 |
| Stage law, K = 3 | 0.756 | 0.782 | 0.813 | 0.847 | 0.898 |
| Stage law, K = 2 | 0.303 | 0.332 | 0.372 | 0.426 | 0.538 |
| Linear superposition | 0.926 | 0.932 | 0.940 | 0.951 | 0.968 |

On ex2 the stage law with the netlist stage count is within 0.04 of the measured peak at every width, where linear superposition is off by 0.17 at the narrowest pulse (Table XIV, Fig. 20(a)). The waveform error at the narrowest pulse is 0.012 for the stage law, from a full-swing fit residual of 0.0066. The stage count matters as Section II-K predicts: K=2 gives 0.30–0.54, far below the measurement.

Table XV: Peak of the stressed gate, inv_chain (fitted at full swing only)

| Pulse width (ps) | 104 | 106 | 111 | 119 | 135 | Full-swing residual |
| --- | --- | --- | --- | --- | --- | --- |
| Measured | 0.879 | 0.935 | 0.972 | 0.992 | 1.003 |  |
| Stage law, K = 7 | 0.701 | 0.832 | 0.913 | 0.961 | 0.987 | 0.0032 |
| Stage law, K = 5 | 0.000 | 0.000 | 0.000 | 0.000 | 0.810 | 0.0037 |
| Stage law, K = 3 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.0141 |
| Stage law, K = 9 | 0.942 | 0.954 | 0.967 | 0.981 | 0.992 | 0.0032 |
| Linear superposition | 0.999 | 1.000 | 1.002 | 1.005 | 1.008 |  |

Fig. 20. Stressed gate of the transistor netlist against the stage law fitted at full swing only, and against linear superposition, at the narrowest, middle and widest of the five pulse widths. (a) ex2. (b) inv_chain.

On inv_chain (Table XV, Fig. 20(b)) the stage law with the netlist stage count follows the measurement at the three widest pulses (within 0.06) and undershoots at the two narrowest, by 0.18 at 104 ps. This is the partial-drive regime in which approximation A2 is active (Section II-O). The other rows show the consequence of the flat residual of Section II-K. The full-swing residuals of K=7, 5 and 3 are 0.0032, 0.0037 and 0.0141; with K=5 the chain extinguishes 4 of the five stressed pulses and with K=3 it extinguishes 5. (Both of those fits end with vt at its upper bound of 0.7, so they are constrained fits, not free ones.) With K=9, at the same residual as K=7, the pulse passes almost unattenuated. The file supplies the band (27); only the stressed measurement can choose within it.

A control establishes that the nonlinearity captured by (19) is a property of the buffers and not of the fit. On io_buf, for input pulses of 1505–2354 ps, every probed predriver node and the pad follow linear superposition (pad peak 0.373 measured against 0.371 predicted at 1505 ps), whereas the same test fails on the output gate of ex2 (0.758 against 0.926) and at the pad of inv_chain (0.490 against 0.853). This statement covers high-going pulses of those widths; the pull-down path of io_buf under much shorter low-going pulses (163–322 ps) is not linear.

### 2) The model's internal gate against the transistor's

The file-only model is judged at the pad; its internal gate GUP is never fitted to anything and the method never observes a gate. On the two buffers whose gate was probed, the gate of the selected file-only build can nevertheless be compared with the real one (Fig. 21, Table XVI).

Table XVI: Peak of the model's internal gate minus the peak of the probed gate, at the five stressed widths (ex2: 810–975 ps; inv_chain: 104–135 ps)

| Buffer | narrowest | 2nd | 3rd | 4th | widest | Waveform rms |
| --- | --- | --- | --- | --- | --- | --- |
| ex2 | -0.053 | -0.050 | -0.042 | -0.036 | -0.025 | 0.06–0.08 |
| inv_chain | -0.079 | -0.061 | -0.020 | -0.009 | -0.007 | 0.04–0.06 |

Fig. 21. The internal gate of the selected file-only model against the probed gate of the transistor netlist, at full swing and at the narrowest and widest stressed pulse. (a) ex2. (b) inv_chain. The model was fitted to the IBIS file and one pad waveform only.

The model's gate has the height and shape of the real one at every width, although nothing in the procedure aimed at the gate: its stressed peak is 0.03–0.05 low on ex2 and 0.01–0.08 low on inv_chain, and it leads the real gate by 56–74 ps on ex2 and 12–20 ps on inv_chain. It is not the same waveform: on ex2 it returns to zero faster than the real gate, and the map absorbs the difference at the pad. The structure of (19) therefore produces an internal gate that resembles the transistor's, while its numbers remain effective parameters and the internals remain approximate.

### 3) Pad level: the complete method on twelve buffers

The complete procedure of Table IV was applied to twelve test buffers: ex2, inv_chain and io_buf, and nine variants of the first two. For each buffer the model was built from its IBIS file and from one transistor-level pad waveform at the pulse width giving about 50 % of the settled swing; Ccomp was estimated from the file as in Table XII. The model was then simulated at five pulse widths (about 50 to 90 % of the settled swing) into the same load and compared with transistor-level simulation of the same buffer. The score is the worst peak error over the five widths, with the peak error defined as in (28). The calibration width is one of the five, so the score is not entirely held out. HSPICE's native IBIS element with the same file is given for comparison.

Fig. 22. Pad voltage for the narrowest stressed pulse of three buffers: transistor-level reference, HSPICE native IBIS, and the proposed model built from the IBIS file and one stressed measurement. The percentages are peak errors against the reference. Time is measured from the input edge.

Table XVII: Pad-level result on the twelve test buffers (worst of five stressed pulse widths)

| Buffer | Netlist K | Selected K | Selected map shape | Proposed: worst peak error (%) | Native IBIS: worst peak error (%) |
| --- | --- | --- | --- | --- | --- |
| ex2 | 3 | 3 | (0.5, 0.7) | 8.3 | 70.7 |
| ex2_base | 3 | 3 | (0.5, 0.7) | 7.0 | no valid run |
| ex2_slowpre | 3 | 3 | (0.5, 0.7) | 5.4 | no valid run |
| ex2_skewp | 3 | 3 | (0.5, 0.7) | 10.0 | no valid run |
| ex2_weak | 3 | 3 | (0.5, 0.7) | 8.7 | no valid run |
| ex2_nomiller | 3 | 3 | (0.5, 0.7) | 7.2 | no valid run |
| inv_chain | 7 | 6 | (0.4, 0.9) | 5.1 | 87.2 |
| inv_base8 | 7 | 7 | (0.4, 0.6) | 8.4 | 66.8 |
| inv_stage4 | 3 | 3 | (0.4, 0.9) | 3.2 | 99.8 |
| inv_skewp | 7 | 7 | (0.4, 0.6) | 8.1 | 37.3 |
| inv_weak | 7 | 7 | (0.4, 0.6) | 4.1 | 35.8 |
| io_buf | 1 | 1 | (0.5, 0.7) | 7.7 | 20.4 |

Fig. 23. Worst stressed peak error at the pad for the twelve test buffers.

The proposed model is within 10 % on all twelve buffers (range 3.2–10.0 %, mean 6.9 %). On the seven buffers for which the native IBIS element produced a valid run, its worst peak error is 20–100 %; on the five ex2 variants it did not produce a valid waveform at any pulse width with the simulation settings used, and no number is reported. The selected stage count equals the netlist count on eleven buffers (inv_chain selects 6 against 7).

### 4) Limits of the evidence

The gate-level test establishes that the structure of (19) extrapolates from full swing to truncated pulses when it is fitted to the true gate; it does not by itself establish the accuracy of the file-only procedure, in which the gate is never observed. That is what the pad-level test addresses, and the two are complementary. Both have a narrow statistical base: the five pulse widths of a buffer sample one stressed trajectory and are strongly correlated, so the evidence rests on the number of buffers. The pad-level test covers single high-going pulses, one load and the typical corner; trains of pulses, low-going pulses (examined so far only on io_buf), other loads and other corners are outside it. The map-shape grid and the band rule (27) were chosen on these same twelve buffers.

One limit is inherited from the device model. Reference [1] states that the α-power law does not reproduce the region near and below threshold, and its delay analysis neglects the opposing device, which is justified for fast inputs [3]. A truncated pulse holds the stages near vt with both devices conducting, which is outside the regime for which [1] validated the model. Equation (19) keeps both devices, but its accuracy there rests on the tests of this section and not on [1].

## O. Summary of approximations

Table XVIII: Approximations of the model

| Label | Approximation | Introduced at | What it removes | Evidence |
| --- | --- | --- | --- | --- |
| A0 | One VTH, α, VD0 for both devices of a stage | (13)–(14) | Per-device parameters | As in [1, Sec. VI]. Table III: the N and P devices differ (x0 0.34 against 0.57 on inv_chain) |
| A1 | p in place of α, with p=1​ | (17) | The measured α (Table III) | Not identifiable from the file (Section II-K) |
| A2 | x0gα/2→xlin, with xlin=0.45​ | (18) | Drive dependence of the saturation boundary | Not identifiable from the file (Section II-K); candidate cause of the inv_chain undershoot in Table XV |
| A3 | K identical normalized stages | (20) | 4(K−1) free parameters | Free fit fails (Section II-K); K selected by (29) |
| A4 | Map shape taken from a grid of three | (22), (26) | Trade-off between gate and map | Selected by (29); grid chosen on the test buffers |

Everything else in (19) is inherited from (1) and (2)–(5): the threshold, the constant-current ramp, the straight-line resistive tail and the two-device structure. Equation (24) is the existing converter's and is unchanged.

## Table I: Notation

| Symbol | Definition | Origin |
| --- | --- | --- |
| u, v​ | Normalized stage input and output, 0 → 1 | (8) |
| vt​ | VTH/VDD; in the fitted model, the effective stage threshold | (9) |
| x0​ | VD0/VDD, saturation boundary at full drive | (9) |
| xlin​ | Effective constant replacing the drive-dependent boundary x0gα/2; 0.45 unless fitted | (18) |
| g(x)​ | Normalized gate drive with cutoff | (11) |
| h(x)​ | Gate factor, gp | (17) |
| r(x)​ | Drain factor, min(1,x/xlin) | (18) |
| sup, sdn​ | ID0/(CVDD) per device, swings per unit time | (16) |
| p​ | Gate exponent, model counterpart of α | (17) |
| K​ | Number of stages | (20) |
| GUP, GDN​ | Normalized gate trajectories of the output pull-up and pull-down devices | (21) |
| M, (vt,map,a,gs)​ | Static gate-to-fraction map and its shape parameters | (22) |
| Ku, Kd​ | Conducting fractions of the output pull-up and pull-down devices | (22), (23) |
| Vfix, Rfix​ | Voltage and resistance of the fixture a V–T waveform was recorded into | Section II-I |
| Ku​ | Ku normalized from its settled off level to its settled on level | (25) |
| e(vt)​ | Relative error of the model's pad peak against the measured peak | (28) |

## References

[1] T. Sakurai and A. R. Newton, “Alpha-power law MOSFET model and its applications to CMOS inverter delay and other formulas,” IEEE J. Solid-State Circuits, vol. 25, no. 2, pp. 584–594, Apr. 1990.

[2] H. Shichman and D. A. Hodges, “Modeling and simulation of insulated-gate field-effect transistor switching circuits,” IEEE J. Solid-State Circuits, vol. SC-3, 1968.

[3] N. Hedenstierna and K. O. Jeppson, “CMOS circuit speed and buffer optimization,” IEEE Trans. Computer-Aided Design, vol. CAD-6, no. 2, pp. 270–280, Mar. 1987.

## Remarks for the manuscript

Sources. Every statement in Sections II-F to II-O was checked against the code and result files of the project; the builder script of this document lists the file behind each. Reference [1] was read in full: equations (2)–(5) here are its (2)–(4), the straight-line triode region of (6) is its own, the single-stage response of Fig. 3(c) is its Appendix B, and approximation A0 is its Section VI. Reference [3] is given as it appears in the reference list of [1]. Reference [2] is given as it appears in the bibliography of Leventhal and Green, Semiconductor Modeling (Springer, 2006); its issue and page numbers were not available for checking and are omitted.

Regenerated numbers. Table XV was regenerated for this document. The earlier committed table had been produced with an explicit-Euler integrator that was later replaced (Section II-G); ex2 (Table XIV) is unaffected to the third decimal for K=3. The search of Fig. 12 and the calibration sweep of Fig. 19 were re-run for this document with the procedure's own code and settings.

Open points. (1) A drive-dependent boundary (15) in place of A2 reduces the inv_chain undershoot at the gate (mean peak error 0.077 to 0.016) but is neutral on ex2 and cannot be identified in the Ku domain, so it has not been adopted; fixing xlin per buffer from a measured x0 (Table III) is untested at the pad. (2) The pad-level score includes the calibration width. (3) Pulse trains, low-going pulses and other loads and corners are not covered by Table XVII. (4) Two of the three Nelder–Mead starts stop in side minima on ex2; a search with more starts, or a global stage, would make step 1 more robust and has not been tried.