# Stage Model Derivation: From the α-Power Law to the Stage Law

October 1, 2026

# II. Stage Model Derivation

This section derives the stage law used in this work from the α-power law MOSFET model [1]. Equations are numbered consecutively; each is obtained from those preceding it, and the approximations are isolated in Section II-G. Notation is collected in Table I.

## A. Node dynamics

A predriver stage is a CMOS inverter whose output node, at voltage Vo, drives a lumped capacitance C. Kirchhoff’s current law at that node gives

C dVodt=IP−IN    1

where IP is the pull-up (PMOS) current into the node and IN is the pull-down (NMOS) current out of it. Equation (1) is the only physical law used; all remaining steps model IP and IN and change variables.

## B. Device model

Each transistor is described by the α-power law [1]. For a device with gate–source voltage VGS and drain–source voltage VDS (magnitudes),

ID=0,D=0 cutoffI′D0V′D0 VDS,VDS<V′D0 triodeI′D0,VDS≥V′D0 saturation    2

with

I′D0=ID0 Dα    3

V′D0=VD0 Dα/2    4

D=clipVGS−VTHVDD−VTH, 0, 1    5

Here ID0 and VD0 are the saturation current and saturation voltage at VGS=VDD, VTH is the threshold, and α∈1,2 is the velocity-saturation index (α=2 recovers the long-channel square law; α→1 for strongly velocity-saturated devices). The lower clip in (5) encodes the cutoff row of (2); the upper clip expresses VGS≤VDD.

Because (2) is piecewise linear in VDS up to V′D0 and constant beyond it, its two conducting rows collapse to a single expression:

ID=I′D0 min1, VDSV′D0    6

Substituting (3) and (4) into (6),

ID=ID0 Dα min1, VDSVD0 Dα/2    7

Equation (7) is the device model in the form used below: a full-drive current, a gate factor Dα, and a drain factor that saturates at unity.

## C. Normalization

Let the stage output move from its resting rail toward its destination rail, a distance VDD. Define, for a rising output,

v≜VoVDD,  u≜VDD−VinVDD    8

so that v is the output’s progress toward its destination and u is the input’s progress toward the level that fully turns on the working (pull-up) device. Both run from 0 to 1 during a transition. Define the normalized device constants

vt≜VTHVDD,  x0≜VD0VDD    9

With the PMOS source at VDD and the NMOS source at ground, the four terminal voltages follow from (8):

VGS,P=u VDD,  VDS,P=1−v VDD,  VGS,N=1−u VDD,  VDS,N=v VDD    10

Substituting (9) and (10) into (5), VDD cancels and the gate drives of the two devices become

DP=gu,  DN=g1−u,  gx≜clipx−vt1−vt, 0, 1    11

Substituting (9) and (10) into the drain ratios of (7),

VDS,PVD0=1−vx0,  VDS,NVD0=vx0    12

Applying (7) to each device with (11) and (12):

IP=ID0,P guα min1, 1−vx0 guα/2    13

IN=ID0,N g1−uα min1, vx0 g1−uα/2    14

Equations (13)–(14) use one vt, α and x0 for both devices; this is the same symmetric-device simplification adopted in [1, Sec. VI], where the two indices are averaged when they differ.

## D. Normalized stage dynamics

From (8), dVo/dt=VDD dv/dt. Substituting this and (13)–(14) into (1) and dividing by C VDD:

dvdt=sup guα min1, 1−vx0 guα/2−sdn g1−uα min1, vx0 g1−uα/2    15

where

sup≜ID0,PC VDD,  sdn≜ID0,NC VDD    16

Equation (15) is exact with respect to (1)–(5). Its parameters are dimensionless except sup and sdn, which have units of swings per second; 1/sup is the time the stage would take to cross the full swing at its full-drive current. At u=1, (15) reduces to a constant-slope ramp for 1−v>x0 followed by an exponential approach with time constant x0/sup=C VD0/ID0,P, which is the CLR3 of [1, App. B].

Note that α appears twice in each term of (15): in the gate factor gα and in the saturation boundary x0 gα/2.

## E. Reduction to the stage law

Two approximations reduce (15) to the form used in this work.

A1 (gate exponent). Replace α in the gate factor by a model parameter p and define

hx≜gxp=clipx−vt1−vt, 0, 1p    17

At full swing the stage input dwells near g=1, where (17) is independent of p; the full-swing waveform therefore cannot identify p, and p=1, the short-channel limit of [1], is adopted as the default.

A2 (fixed saturation boundary). Replace the drive-dependent boundary x0 gα/2 in (15) by its full-drive value, a constant xlin, and define

rx≜min1, xxlin,  xlin=x0    18

At g=1, (18) coincides with the boundary in (15); the two differ only under partial drive.

Applying (17) and (18) to (15) yields the stage law:

dvdt=sup hu r1−v−sdn h1−u rv    19

Comparing (19) with (15) term by term: sup and sdn are unchanged; hu replaces guα (A1); r⋅ replaces the drive-dependent drain factor (A2). The second appearance of the gate drive in (15), inside the boundary, is the only structural element removed.

For a falling output, (8) becomes v=VDD−Vo/VDD and u=Vin/VDD; (10) exchanges the P and N rows, and (19) is unchanged.

## F. Chain model and the gate trajectory

The predriver is modeled as K stages, each obeying (19) with one shared parameter set {sup,sdn,vt,xlin,p} (approximation A3, identical normalized stages). The stages are coupled by identifying each stage’s output with the next stage’s input, which (8) makes possible because every node is expressed as its own progress from 0 to 1. The first stage is driven by a comparator on the buffer’s input pin:

u1t=1Vint>12VDD,  uk+1t=vkt, k=1,…,K−1    20

The last stage’s output is the trajectory of the output device’s gate, normalized from off to on:

GUPt≜vKt,  GDNt≜1−GUPt    21

GUP drives the pull-up device of the output stage and GDN its pull-down; (21) treats the two as complementary. [CONFIRM: the complementary relation is taken from the implementation; the physical justification for it has not been stated and should be added by the authors.]

Equations (19)–(21) are integrated with Heun’s method (second-order Runge–Kutta) at 1 ps substeps, with each state clipped to −0.05,1.05 to guard against overshoot. The step size matters: explicit Euler at 2 ps was found to bias the fastest stages (time constants near 20 ps), passing a 119 ps pulse that the same chain in ngspice extinguished, whereas Heun agrees with ngspice.

For circuit simulation, each stage of (19) is emitted as a behavioral current source driving an explicit capacitor, so that the normalized node vk is a real SPICE node:

ISTG,k=−Cg⋅109sup hvk−1 r1−vk−sdn h1−vk−1 rvk,  Cgdvkdt=−ISTG,k    22

The factor 109 converts slopes given per nanosecond to per second; Cg cancels from the resulting dvk/dt, so its value is immaterial and it serves only to give the node a state.

## G. From the gate trajectory to Ku and the pad current

The gate trajectory is not observable from an IBIS file. What the file encodes is the output device’s conducting fraction Kut during one complete transition, and Ku is obtained from the gate by a static, MOSFET-shaped map:

Kut=M​GUPt,  Mg≜clipg−vt,mapgs−vt,map, 0, 1a    23

Here vt,map is the gate level at which the output device begins to conduct, a is the curvature of its turn-on, and gs≤1 allows Ku to reach unity before the gate reaches its rail. The parameters of (23) describe the output device and are distinct from the stage parameters of (17), which describe the predriver stages; both have the same clip-and-power form because both are gate-drive factors of the type (3), but they play different roles and must not be confused. The pull-down fraction is obtained in the same way, Kdt=MdGDNt, with its own map.

The pad current then follows the IBIS convention: the conducting fractions scale the file’s own fully-on I–V tables. [CONFIRM: (24) is written in the standard IBIS form; verify that it matches the converter’s actual pad-current expression, including the clamp terms.]

Ipadt=Kut IPU​Vpad+Kdt IPD​Vpad+Iclamp​Vpad    24

Equation (24) is unchanged from the existing converter; the proposed method replaces only the source of Kut and Kdt. For a full transition, (19)–(24) reproduce the file’s waveform to within the fit residual of Section II-H. For a truncated pulse, the input u1 of (20) reverses before the chain has settled; the chain then produces a gate trajectory that turns back partway, (23) reads Ku off that trajectory, and (24) delivers a pad current that no full-swing table contains. This is the mechanism by which the method extends a full-swing IBIS file to short pulses.

Equation (23) is the source of the identifiability problem addressed in Section II-I. Because Ku=MGUP is a composition, any monotone trajectory G paired with the map M∘GUP∘G−1 reproduces the same Kut exactly. A full transition pins the composition and says nothing about the factors; a truncated pulse depends on the factors. The stage law supplies the family of admissible trajectories, and the map shape must be fixed before the stage parameters are fitted, otherwise (19) and (23) trade off against one another.

## H. Fitting the stage parameters to the IBIS file

The fitting target is the gate-driven part of the file’s own Kut. It is obtained by simulating the shipped (gate-state) IBIS subcircuit of the same buffer through one full-swing transition and probing its internal Ku node, direction-selected and with the residual transient excluded. Both model and target are normalized from rest to on,

Kut=clipKut−KutrestKuton−Kutrest, 0, 1,  trest=4.5 ns, ton=12 ns    25

With the map shape vt,map,a fixed and a candidate stage count K, the stage parameters are chosen to minimize the root-mean-square mismatch over a 4–21 ns window sampled at 2 ps:

{sup, sdn, vt}⋆K=argmin rmstM​vKt; θ, K−Kufilet,  xlin, p fixed    26

During the fit the chain is driven by an ideal 10 ns box beginning at ton=5.0 ns+tedge/2 rather than by the comparator of (20). The minimization uses Nelder–Mead with three restarts (s0=2,8,30 per ns, 800 iterations each) under the bounds 0≤vt≤0.7 and, when free, 0.02≤xlin≤1.5, and is repeated for every candidate K. The comparison in (26) is made in the Ku domain because the gate is unobservable; the model gate is pushed through (23) and only then compared.

The pull-down fraction requires its own domain. For an open-drain part, fitting Kd through the Ku target leaves its onset unconstrained, because the target is zero wherever g<vt,map, which is exactly where Kd turns on; this produced a 250 ps early onset on one test buffer. Kd is therefore fitted through its own map with GDN=1−GUP.

Worked example. For buffer ex2 (3.3 V, 0.6 µm process) with K=3, map shape 0.57,0.64 and xlin=0.45 fixed, (26) gives sup=2.190 per ns, sdn=2.124 per ns, vt=0.528 at a Ku-domain rms of 0.023. Three numbers fitted to a curve the file already contains; the stressed measurement is not consumed here.

## I. Identifiability: what the full-swing file determines

Equation (26) determines the rates well and the structure poorly. Table II lists the fit residual of (26) for each K independently on three of the twelve test buffers; the transistor-level netlist of each test buffer is available, so the true stage count is known.

Table II: Ku-domain rms of (26) versus stage count K

| Buffer | K=1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | True K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ex2 | 0.0829 | 0.0292 | 0.0233 | 0.0224 | 0.0215 | 0.0222 | 0.0235 | 0.0255 | 0.0274 | 0.0294 | 3 |
| inv_chain | 0.0468 | 0.0216 | 0.0141 | 0.0108 | 0.0076 | 0.0071 | 0.0070 | 0.0072 | 0.0072 | 0.0074 | 7 |
| io_buf PU | 0.0176 | 0.0264 | 0.0500 | 0.0639 | 0.0724 | 0.0783 | 0.0825 | 0.0858 | 0.0883 | 0.0904 | 1 |

Two features are evident. First, K=1 fails on ex2 and inv_chain, because a single stage cannot produce the required delay with the required edge rate; the full swing does constrain the slope. Second, beyond that the residual is flat: K=3 through 6 lie within 0.002 on ex2, and K=6 through 9 differ in the fourth decimal on inv_chain. The best-fitting K equals the netlist count only where the curve has a true minimum (io_buf, and marginally inv_chain). This is the composition degeneracy of Section II-G appearing quantitatively, and it has a direct reading in (15): the stage count, the exponent p, and the boundary all act only through g<1, which a full-swing input passes through briefly.

A free fit with all 4K parameters is not merely uninformative but misleading. On ex2 with K=2 and every parameter free, the full-swing residual falls to 0.0046, yet the fit achieves this by splitting the delay into one very slow stage and one fast one, and such a chain extinguishes short pulses: it predicts a stressed gate peak of 0.228 against a measured 0.758. Shared parameters (A3) remove this corner and leave K as the single structural unknown.

The procedure therefore retains a band of stage counts rather than a point estimate:

K=the three smallest K with rmsK≤1.25 minK′rmsK′    27

On the twelve test buffers, (27) contains the netlist stage count in every case.

## J. The single stressed measurement

One stressed measurement, a pad waveform for one short pulse of known width, settles what the file cannot. It is used twice: to choose the stage count within K, and to recalibrate the threshold vt by bisection with all other parameters held at their values from (26). [CONFIRM: the exact selection procedure for K (what is simulated and compared) and the objective minimized in the vt bisection are not stated in the source documents; (28) below is written as a waveform-rms objective and must be checked against the recipe.]

vtcal=argminvt rmstVpadmodelt; vt−Vpadmeast,  K∈K fixed    28

For ex2, calibration on an 810 ps pulse moved vt from 0.528 to 0.487. The selection is made on the whole waveform rather than on the peak alone, since ranking by peak was measured to be substantially less reliable. [CONFIRM: the recipe reports 217.3 mV against 52.3 mV for this comparison; the quantity measured should be stated.] Because a chain of (19) extinguishes a pulse once it drops below vt at any stage, vt and K jointly determine where the pulse-swallowing cliff lies, which is why neither can be left to the full-swing fit.

## K. Verification

The claim to be verified is that (19), fitted to a full-swing transition only, predicts the gate’s response to a truncated pulse. The test uses buffers with transistor-level netlists: the stage parameters are fitted at full swing against the probed internal gate node (node n4 for ex2 [CONFIRM: node for inv_chain]), the chain is then driven with the measured stressed input, and its output is compared with the measured stressed gate. No stressed data enters the fit. Linear superposition of the full-swing step response serves as the comparison. [CONFIRM: the source documents rule out RC cascades, delay-plus-RC, and superposition as separate structures; only superposition is tabulated here.]

Table III: Stressed gate peak, ex2 (K=3, fitted at full swing only)

| Pulse width (ps) | 810 | 830 | 858 | 895 | 975 |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.758 | 0.797 | 0.838 | 0.880 | 0.936 |
| Stage law, K=3 | 0.755 | 0.782 | 0.813 | 0.847 | 0.898 |
| Linear superposition | 0.926 | 0.932 | 0.940 | 0.951 | 0.968 |

The stage law is within 0.05 at every width; linear superposition is off by 0.17 at the narrowest pulse. The stressed waveform rms is 0.012 for the stage law against 0.069 for superposition, from a full-swing fit residual of 0.0066. The stage count matters as Section II-I predicts: K=2 yields 0.29–0.52 at the same widths, swallowing the pulse.

Table IV: Stressed gate peak, inv_chain (fitted at full swing only)

| Pulse width (ps) | 104 | 106 | 111 | 119 | 135 |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.879 | 0.935 | 0.972 | 0.992 | 1.003 |
| Stage law, K=7 | 0.701 | 0.832 | 0.913 | 0.961 | 0.987 |
| Stage law, K=5 | 0 | 0 | 0 | 0 | 0.869 |
| Stage law, K=3 | 0 | 0 | 0 | 0 | 0 |

The full-swing residuals for K=7,5,3 are 0.0032, 0.0037 and 0.0141: K=5 is within 16 percent of K=7 at everything the file can see, and extinguishes four of the five stressed pulses. The file supplies the band of (27); only the stressed measurement of Section II-J can choose within it. The K=7 prediction undershoots at the narrowest widths (by 0.18 at 104 ps), which is the partial-drive regime where approximation A2 is active; this is the case for which a drive-dependent boundary (Section II-E) is under evaluation. [MISSING: the K=7 row is from the current code; the K=5 and K=3 rows are from a committed run that no longer reproduces and must be regenerated before use.]

A control establishes that the nonlinearity captured here is a property of the buffers and not of the fit. On io_buf, whose pull-up predriver is a single stage and which the source documents describe as linear end to end [CONFIRM: the same documents report that io_buf’s pull-down chain extinguishes short pulses, which is nonlinear; the scope of the linearity claim needs to be stated precisely], the linear structures are exact; RC cascades, delay-plus-RC, and superposition of step responses are ruled out on ex2 and inv_chain, and the shipped model is built from exactly those.

Two limits of this verification should be stated. It is performed on the probed gate node of a transistor-level netlist, which establishes that the structure of (19) extrapolates from full swing to truncated pulses when the true gate is available; it does not by itself establish the accuracy of the complete file-only pipeline, (26)–(28) followed by (23)–(24), at the pad. That end-to-end accuracy is assessed separately at the pad on the twelve test buffers. [MISSING: pad-level, file-only results and the reference they are compared against; the source documents mention an existing ±10 % criterion but give no table.] And the five widths per buffer are samples of one stressed trajectory, so the evidence rests on the number of buffers tested rather than on the number of widths.

## L. Summary of approximations

| Label | Approximation | Introduced at | What it removes | Testable by |
| --- | --- | --- | --- | --- |
| A0 | One VTH, α, VD0 for both devices | (13)–(14) | Per-device parameters | Device extraction |
| A1 | p for α, p=1 | (17) | Measured α (1.1–1.4 here) | Stressed data only |
| A2 | x0 gα/2→xlin | (18) | Drive dependence of the boundary | Stressed data only |
| A3 | K identical stages | (20) | 4K−1 free parameters | Stressed data (choice of K) |
| A4 | Map shape vt,map,a fixed before the fit | (23), (26) | Trade-off between gate and map | Pad-level results across buffers [CONFIRM: A4 added by the drafter, not from source documents] |

Everything else in (19) is inherited unchanged from (1) and (2)–(5): the threshold, the constant-current ramp, the straight-line resistive tail, and the two-device structure. Equation (24) is inherited unchanged from the IBIS convention.

## Table I: Notation

| Symbol | Definition | Origin |
| --- | --- | --- |
| u, v | Normalized stage input and output, 0 → 1 | (8) |
| vt | VTH/VDD | (9) |
| x0 | VD0/VDD, saturation boundary at full drive | (9) |
| xlin | Fixed value replacing x0 gα/2 | (18) |
| gx | Normalized gate drive with cutoff | (11) |
| hx | Gate factor, gp | (17) |
| rx | Drain factor, min1, x/xlin | (18) |
| sup, sdn | ID0/C VDD per device, swings/s | (16) |
| p | Gate exponent, model counterpart of α | (17) |
| K | Number of stages | (20) |
| GUP, GDN | Normalized gate trajectories of the output pull-up and pull-down devices | (21) |
| M⋅, vt,map,a,gs | Static gate-to-Ku map and its parameters | (23) |
| Ku, Kd | Conducting fractions of the output pull-up and pull-down devices | (23) |
| Ku | Ku normalized from rest to on | (25) |
| K | Band of admissible stage counts | (27) |

## References

[1] T. Sakurai and A. R. Newton, “Alpha-power law MOSFET model and its applications to CMOS inverter delay and other formulas,” IEEE J. Solid-State Circuits, vol. 25, no. 2, pp. 584–594, Apr. 1990.

[2] [CONFIRM bibliographic details] H. Shichman and D. A. Hodges, “Modeling and simulation of insulated-gate field-effect transistor switching circuits,” IEEE J. Solid-State Circuits, vol. SC-3, no. 3, pp. 285–289, Sep. 1968.

[3] [CONFIRM bibliographic details] N. Hedenstierna and K. O. Jeppson, “CMOS circuit speed and buffer optimization,” IEEE Trans. Comput.-Aided Design, vol. CAD-6, no. 2, pp. 270–281, Mar. 1987 (the fast-input condition under which [1] neglects the opposing device).

## Remarks for the manuscript

First, if the moving-boundary variant is ever adopted, it is simply (15) with A1 only, so Sections II-E through II-K need no restructuring; only (18) changes. Second, the statement that the full-swing waveform cannot identify p, the boundary, or K is the claim reviewers will probe; Table II is the quantitative support, and the structural reason is that all three enter (15) only through g<1. Third, the pad-level end-to-end results belong in Section II-K; the gate-level verification there establishes the model structure, and the pad-level table establishes the method.